| Sunrise Pinlist / Padring |  |  |
| --- | --- | --- |
| Project developed by: SLICE Semiconductor Project funded by: nlnet foundation Document ID: Sunrise Pinlist * Padring Created by: Diarmuid Collins Date created: 26/06/2026 Date modified: Revision: D |  |  |
| Valid from | 18.03.2025 |  |

Table of Contents

1	General information	2

2	Pinlist	2

3	Padring	3

4	Revision History	4

# General information

This document defines the top level pin list and padring for Sunrise.

# Pinlist

Full pinlist of the ASIC is shown in Table 1.

| # | Name | Description | IO Type | Domain | Value |
| --- | --- | --- | --- | --- | --- |
| 1 | SPI_CPOL | Mode select (or rdac_ctrl[1]) | IP | Digital | 0 / vdda_hv |
| 2 | SPI_CPHA | Mode select (or rdac_ctrl[0]) | IP | Digital | 0 / vdda_hv |
| 3 | I2C_SCL | I2C serial clock lin (or cdac_sel[4]) | IP | Digital | 0 / vdda_hv |
| 4 | I2C_SDA_O | I2C data out | TRI | Digital | 0 / vdda_hv |
| 5 | I2C_SDA_I | I2C data in (or cdac_sel[3]) | IP | Digital | 0 / vdda_hv |
| 6 | SPI_nCS | SPI chip select (or INA pd) | IP | Digital | 0 / vdda_hv |
| 7 | SPI_SCLK | SPI serial clock line (or int bias) | IP | Digital | 0 / vdda_hv |
| 8 | SPI_MOSI | SPI MOSI data line (or ip_ctrl[1]) | IP | Digital | 0 / vdda_hv |
| 9 | SPI_MISO | SPI MISO data line | TRI | Digital | 0 / vdda_hv |
| 10 | SPI_MISO_CTRL | SPI MISO tri-stat ctrl bypassl (or ip_ctrl[0]) | IP | Digital | 0 / vdda_hv |
| 11 | CLK_EXT | External clock | IP | Digital | 0 / vdda_hv |
| 12 | RO_OUT | RO output | OP | Digital | 0 / vdda_hv |
| 13 | CLK_INT_EN | Pass RO o/p to digital | IP | Digital | 0 / vdda_hv |
| 14 | RO_EN | Enable the RO | IP | Digital | 0 / vdda_hv |
| 15 | VTSNS2_N | Temp sensor bias | ANA | Analog | 1 / 10uA |
| 16 | VTSNS2_P | Temp sensor bias | ANA | Analog | 1 / 10uA |
| 17 | VREF | Voltage VOUT is referred to | ANA | Analog | 0 – Vref_max |
| 18 | VOUT | INA output | ANA | Analog | 50mV – 3.3-50mV |
| 19 | VSSD | Digital ground | Supply | Digital | 0V |
| 20 | VDDA_LV | Low voltage supply | PWR | Digital | 1.2 – 1.5V |
| 21 | IBIAS_IN_5U | External bias | ANA | Analog | 5uA |
| 22 | INT_BIAS | Sense int_bias internal signal | OP | Digital | 0 / vdda_hv |
| 23 | PORB | Power on reset (active lo) | IP | Digital | 0 / vdda_hv |
| 24 | SERIAL_SEL | Select between SPI and I2C (or cdac_sel[5]) | IP | Digital | 0 / vdda_hv |
| 25 | TEST_MODE | Repurpose SPI / I2C controls as direct i/p’s | IP | Digital | 0 / vdda_hv |
| 26 | VSSA | Analog ground | GND | Analog | 0V |
| 27 | VDDA_HV | High voltage supply | PWR | Analog | 3.3V |
| 28 | VSNS_EXT_N1 | External sense (-ve) | ANA | Analog | 0.9 – Vcm_max_n |
| 29 | VSNS_EXT_P1 | External sense (-ve) | ANA | Analog | 0.9 – Vcm_max_n |
| 30 | VTSNS1_N | Temp sensor bias | ANA | Analog | 1 / 10uA |
| 31 | VTSNS1_P | Temp sensor bias | ANA | Analog | 1 / 10uA |
| 32 | VSNS_EXT_N2 | External sense (-ve) with 260Ω CDM protection | ANA | Analog | 0.9 – Vcm_max_n |
| 33 | VSNS_EXT_P2 | External sense (+ve) with 260Ω CDM protection | ANA | Analog | 0.9 – Vcm_max_p |
| 34 | I2C_ADDR[0] | Address (or cdac_sel[0]) | IP | Digital | 0 / vdda_hv |
| 35 | I2C_ADDR[1] | Address (or cdac_sel[1]) | IP | Digital | 0 / vdda_hv |
| 36 | I2C_ADDR[2] | Address (or cdac_sel[2]) | IP | Digital | 0 / vdda_hv |

Table 1 Top level pinlist

As shown, the ASIC will occupy 35 pins + 1 spare. The spare pin is included as final ASIC will fit into a 36QFN package (provided by Serma).

As shown, VSNS_EXT_P/N inputs can vary between 0.9V and Vcm_max_p/n where:

Vcmmaxp= vddahv-50mV-Avnia∆vin…(1)

Vcmmaxn= Vcmmaxp- ∆vin …(2)

As shown, VREF can vary between 0 and Vref_max where:

Vrefmax=vddahv-50mV-Avina∆vin…(3)

Note: In (1), Avnia = gain of the non-inverting loop. In (3), Avina = gain of the INA. In (1) and (3), 50mV represents the margin from the supply rails which vout can swing.

All the digital pins (apart from the Tristate ones) can be repurposed as direct control inputs. The direct control corresponding to each input is shown in italics in the description.

All digital inputs will use the “digital_in” IO.

All digital outputs will use the “digital_out” IO.

All digital tristate pins will use the “digital_io” IO.  The c2p_en inputs to these IO’s will be controlled by the digital logic.

# Padring

Padring is shown in Figure 1.

Note that pin locations are driven mainly by layout floorplan.

Figure 1 Sunrise padring

# Revision History

| Rev. | Date | Author | Changes, Remarks |
| --- | --- | --- | --- |
| A | 13.06.2026 | Diarmuid Collins | Created |
| B | 22.06.2026 | Diarmuid Collins | Re-arranged pinlist / padring |
| C | 23.06.2026 | Diarmuid Collins | Re-arranged ro_en / spare pins to avoid antenna violations |
| D | 26.06.2026 | Diarmuid Collins | Re-arranged digital pins / corrected typos |

Table 2: Revision history